Roma ACC - Tunis ACC

V 2606

Sommario

Purpose	2

ACC units description	2

Rome ACC	2

Tunis ACC	3

Frequencies	3

Roma ACC	3

Tunis ACC	4

General Procedures	4

Enroute Coordination	5

LIRR->DTTC	5

DTTC->LIRR	6

Military areas coordination and management	7

Validity and Revision	7

# Purpose

The purpose of this Letter of Agreement (LoA) is to define the coordination procedures to be applied between Tunis ACC and Roma ACC when providing ATS to GAT operating in IFR/VFR. The content of this LoA is approved by the concerned FIR Chiefs and its application is mandatory for all IVAO members operating on concerned position.

# ACC units description

## Rome ACC

The ATS unit in charge of the Roma FIR and UIR is Roma Radar and consists in three primary sectors (NE, EW, SU). LIRR_SU_CTR might be opened in two secondary sectors (LIRR_SU_CTR and LIRR_ES_CTR).

Roma Radar shall announce its actual configuration to Tunis Control as soon as possible.

## Tunis ACC

The ATC unit in charge of FIR and UIR of Tunis ACC is Tunis Control and consists of 3  sectors that are oftentimes grouped into 1 main sector.

# Frequencies

## Roma ACC

| ATC Position | Callsign | Frequency |
| --- | --- | --- |
| Roma Radar | LIRR_SU_CTR | 128.800 MHz |
| Roma Radar | LIRR_EW_CTR | 127.125 MHz |
| Roma Military | LIRR_MIL_CTR | 118.750 MHz |
| Cagliari Radar | LIEE_APP | 118.400 MHz |
| Cagliari Radar | LIEE_MIL_APP | 125.950 MHz |

For all CTR sectors: SFC- UNL

## Tunis ACC

| ATC Position | Callsign | Frequency |
| --- | --- | --- |
| Tunis Radar Boundary with LIRREW/SU | DTTC_CTR | 132.550 MHz |
| Tunis Radar Boundary with LIRREW/SU | DTTC_N_CTR -FL325 | 132.550 MHz |
| Tunis Radar Boundary with LIRREW/SU | DTTC_U_CTR FL325+ | 129.500 MHz |
| Tunis Radar (no boundary with LIRR) | DTTC_S_CTR | 129.300 MHz |
| Tunis Information | DTTC_FSS | 128.900 MHz |

For all CTR sectors: SFC- UNL

# General Procedures

Traffic in sequence shall be handed over with minimum spacing of 10 NM. This separation must be constant (aircraft restrained to the same speed) or increasing (succeeding aircraft is not faster). Coordination of speed control should be granted via radar labels and does need the approval and acknowledgement from the receiving sector.

Traffic shall be handed over as soon as practicable and whenever possible, at least 3000 ft before reaching the cleared flight level. If the transfer point is not defined in this LoA, at least 10 Nm before the airspace limits.

All traffics shall be handed over properly separated and clear of any conflict. Unless the receiving ATC unit issue a clearance modifying the route, altitude or speed, the transferring sector remains responsible for separation.

# Enroute Coordination

## LIRR->DTTC

Roma ACC will transfer aircraft cleared to ODD flight levels, as RFL, except as provided in the following table.

| FIX | Flight Level | Remarks |
| --- | --- | --- |
| NOLSI | 250 | DEST DTTA TUC/MANEX/MARSA possible |
| ABDAB | 230 |  |
| SUSIP/TUPAL | 290 |  |
| TUPAL | 290 | DEST DTNH NBA/ATLEL/MOVAR possible |
| TUNEX | 310 |  |

## DTTC->LIRR

Tunis ACC will transfer aircraft cleared to EVEN flight levels, as RFL, except as provided in the following table.

| FIX | Flight Level | Remarks |
| --- | --- | --- |
| OSMAR | 260 320 | ARR LIEE DEP DTTA CAR/CORSI/ELB/BETOT LATAN.. possible |
| NIBEL | 280 |  |
| SUSIP |  |  |
| TUPAL |  |  |
| BABLO | 300 | DEP DTMB |
| TUPAL/BABLO | 320 or BELOW | DEST LICJ → FL190 MEGAN/TRP possible DEST LICT → FL150 MEGAN/LOPKO/TRP/ MORUX/EKTOR possible |

Tunis ACC and Roma ACC coordinate actively for DCTs within respective sectors and beyond.

The segment NIVAB-TABOT is delegated to Tunis ACC and traffic transfer is LFMM-DTTC-LFMM. The French (LFMM) are often misinformed about this and neglect any coordination in case of event or just everyday use.

# Military areas coordination and management

NIL

# Validity and Revision

This vLoA is effective as of the effective date until a future revision.

If inconsistencies are found between this document and the national aeronautical legislation, the corresponding fixes, airways, limits, minimums, etc. in force in AIP Italy must be considered. If such correspondences do not exist, it will be necessary to proceed by applying occasional derogations.